SPECIFICATION SHEET FOR ROTORK GEARS IB BEVEL, IS SPUR AND IW WORM GEAR VALVE OPERATORS SUITABLE FOR BURIED SERVICE

For buried service, Rotork Gears gearboxes are grease filled with Renolit CL-X2 (old name: Lacerta CL2X), to give an operating temperature range of -40°C to +120°C and are totally sealed to give a weatherproof construction to IP67.

The gearbox input shafts are bright zinc plated.

All fasteners are bright zinc plated.

On worm units, the exposed output sleeve retaining bolts are protected typically in accordance with drawing IW6C91.

Gearcases, baseplates and input housings are cast iron to BS1452 grade 250 or SNG iron to BS2789 grade 420/12.

All gearboxes are finished with PA10 primer suitable for overcoating with most conventional paints, polyurethanes, epoxies and chlorinated rubber paints.

These conditions allow buried service to a depth of 15 metres.