SPECIFICATION AND OPTIONS FOR ROTORK GEARS BEVEL GEAR OPERATORS

Specification:

-Output torque range available up to 10850Nm (manual) and 8150Nm (motorised).

-Thrust range available up to 1557kN.

-Valve spindle acceptances up to Ø120mm (bored and keywayed) and Ø150mm ( threaded).

-Basic gear ratios from 1:1 to 8:1.

-Compatible with auxiliary spur gear attachment AS type giving total ratios up to 120:1.

Options:

-Auxiliary spurs may be fitted with `twin input’ shafts to give high and low speed operation.

IB/AS Combination Bevel/Auxiliary Spur Gearbox

-Form B non thrust taking units are available.

-Through input shafts are available for operation of secondary gearboxes on twin spindle applications.  Input options include:-

Manual both ends		           Motorised / Manual		         Motorised both ends

-Valve spindle cover tubes are available in steel and clear polycarbonate.

-Valve position indication is available via several types of gearbox mounted indicator including:-

Rising indicators	        	         Rising indicators		     	   Dial type indicators

For rising and non-rising stems                (polycarbonate)        	        for rising and non-rising stems

(steel)

-Steel indicators are available with limit switches for end of travel indication.

-Middle gearboxes MB type are available with two or three shafts to drive further gearboxes at various angles.

Other options available:

-Standard rotation is clockwise output for clockwise input.  Anti-clockwise output for clockwise input is available.

-Buried duty, Submersible duty, Marinised duty, Raw Sewage duty, Nuclear duty, High  and low Temperature duty.

-Lockable hand wheels and flanges, Temperature and Thrust Compensators

Standard rising spindle cover tubes are available in 200mm increments (100mm increments up to 1 metre long).  Alternative lengths are available on request.  Customer specific covers available include:-

a)  Aramco  b)  drilled / tapped holes in end plates  c)  NPT thread forms.