PDM EDD for ER278

Installation Description Documentation

| Customer: Contact person: Project No.: Project Manager: |  | Rotork Controls Ltd Tom Hannon 17400a Hanspeter Scherr |  | DocName | input.doc |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DocPages | 9 |  |
|  |  |  |  | DocType |  |  |
|  |  |  |  | FNr. Autor: | 3 ju |  |
| Rev. | Changes |  | edit (mm.tt.jj/KZ) | checked (mm.tt.jj/KZ) |  | released (mm.tt.jj/KZ) |
| 0.1 | Initial version |  | 02.02.04/JR |  |  |  |
| 0.2 | Update to pdm 6.x |  | 14.09.2009/jr |  |  |  |

Contents

1 Introduction	3

# Introduction

Start SIMATIC MANAGER STEP7 VERSION 5.3 with PDM 6+SP5

Start in this manager the "Manage Device Catalog"

Select the path of DD disk or DD Cdrom or your location file.

Please select (for example) the path directory

“C:\Temp\Rotork_PDM_V4_1”

Select in this dialog the Rotork devices.

After then the following message displays.

Then insert a new project by the menue FileNew and insert for your project a new Name.

After then choose in the menu View  Process Device Network View.

Insert PC and PROFIBUS DP with the context menu on the right mouse.

Please, insert in the PROFIBUS DP-Network  a new PROFIBUS PD device.

Insert in the name field "Profibus Rotork 2".

Please enter the DP-Address of your device.

Doubleclick on the Icon "Profibus Rotork 2".

Then a dialog with installed DP Devices will open.

Please select your device in ActuatorsElectricRotorkProfibus Mk2.